PI

- User: bina

- password: binacam

- http://<192.168.1.206>:8070/stream.mjpg

LEDs

- longer wire is positive

- Voltage: 3v

- Model: 0402 0603

TMC2209

- reduction ratio of 1:9.5

- stepping angle of 18 degrees

- Motor phase resistance: about 9.6 ohms

- @5V short circuit current: 0.49A

Motor Pinout

LED circuit

GPIO21 → 2x330Ω resistors → LED+ → LED- → common ground

Motor circuit

- Power (NOT FROM PI)

  - PI: MiniUSB port

  - Motor driver (NOT from rpi) VM

  - Shared ground

- Motor → TMC2209

  - A+ → M1A			(ORANGE)

  - A− → M1B			(YELLOW)

  - B+ → M2A			(BLACK)

  - B− → M2B			(BROWN)

- Pi → TMC2209

  - Power → VM

  - Pi 3.3v → VIO

  - Pi GND → GND

  - Cont. GND → Pi GND

  - GPIO14 → 1kohm → USART

  - GPIO18 → STEP

  - GPIO23 → DIR

  - GPIO24 → EN

Sudo raspi-config -> interface option -> enable serial

sudo systemctl stop dhcpcd

sudo systemctl stop NetworkManager

sudo wpa_supplicant -i wlan0 -D nl80211 -c /etc/wpa_supplicant/wpa_supplicant-wlan0.conf

wpa_cli p2p_find

ape_cli p2p_connect 2a:c2:1f:97:e1:7c pbc